| Issued To | : | Site Contractor |  | Date of Issue | : | Novemeber |
| --- | --- | --- | --- | --- | --- | --- |
| Copy To | : | Project Manager, QA/ QC Manager |  | NCR Ref. | : | NCR-2025-001 |
| Subject | : | Cold joint at slab edge |  | Project | : | [Project Name] |
| Person Responsible | : | Site Foreman |  | Cost Impact | : | To be assessed |

| DESCRIPTION OF NON-CONFORMANCE |  |  |  |  |  |  |  |  | CORRECTION |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QMS Clause Ref: 7.5.3 (Concrete Works) |  |  |  |  |  |  |  |  | Use as it is Repair / Re-work Replace Item Reject / Scrap Use with Client Permission Others (Specify) |  |  |
| Cold joint observed at slab edge during inspection. The joint indicates improper concreting sequence or delay between concrete pours, resulting in insufficient bonding between concrete layers |  |  |  |  |  |  |  |  |  |  |  |
| Identified By: QC inspector |  | Department: Quality Control |  |  |  | Date: November 4, 2025 |  |  | Proposed By : |  |  |
| ROOT CAUSE OF NON-CONFORMANCE |  |  |  |  |  |  |  |  |  |  |  |
| - Delay in concrete pour timing exceeded the initial setting time - Inadequate planning of concrete delivery schedule - Insufficient coordination between batching plant and site team - Surface preparation not performed before subsequent pour |  |  |  |  |  |  |  |  |  |  |  |
| Answered By : Construction Manager |  |  |  |  | Department : Construction |  |  |  |  | Date Identified : |  |
| CORRECTIVE ACTION |  |  |  |  |  |  |  |  |  |  |  |
| 1. Remove loose concrete at cold joint location (minimum 50mm depth) 2. Clean surface with wire brush and high-pressure water jet 3. Apply bonding agent as per manufacturer specifications 4. Pour new concrete with proper compaction and curing 5. Implement concrete delivery schedule with backup suppliers 6. Ensure surface preparation checklist is completed before each pour |  |  |  |  |  |  |  |  |  |  |  |
| Proposed By | Proposed Date |  |  | Target Date |  |  | Action For | Department |  |  | Start Date |
| Site Engineer | November 10, 2025 |  |  |  |  |  |  |  |  |  |  |
| VERIFICATION & EFFECTIVENESS OF CORRECTIVE ACTION |  |  |  |  |  |  |  |  |  |  |  |
| Corrective action completed and verified through: - Visual inspection of repaired area - Core sample testing for bond strength - Review of updated concreting procedure |  |  |  |  |  |  |  |  |  |  |  |
| Verified By: QA/QC Manager |  |  | Date [Upon completion] |  |  |  | Reviewed By : |  |  |  | Date : |